Arduino Lesson 4

Advertising light

We often see some billboards, composed of all sorts of color of the led lamp constantly change. This experiment will show you the basic effects.

1）Parts require

Leds：6

220Ω Resistor：6

Jumper Wires：1 Bundle

2）Connection

Put 6 leds on pins in order as below

- Program for reference

int BASE = 2 ;  //First LED I/O contact

int NUM = 6;   //LED Numbers

void setup()

{

for (int i = BASE; i < BASE + NUM; i ++)

{

pinMode(i, OUTPUT);   // Setup digital I/O as output

}

}

void loop()

{

for (int i = BASE; i < BASE + NUM; i ++)

{

digitalWrite(i, LOW);    // Setup digital I/O output as “low” which means turn on the light gradually

delay(200);        //Delay

}

for (int i = BASE; i < BASE + NUM; i ++)

{

digitalWrite(i, HIGH);    //Setup digital I/O output as “low” which means turn on the light gradually

delay(200);  //Delay

}

}