Arduino Lesson 12

LM35 Temperature Sensor

LM35 is a common and east to use temperature senor. Only one LM35 required in electronic applications, just use a analog interface. The difficult part is how to exchange the analog value to actual temperature by arithmetic.

- Parts Required

- LM35 x 1

- Breadboard x 1

- Jumper wires x 1 Bundle

- Connect-it-up

- Program for reference

int potPin = 0; //Set analog interface #0 accessed to LM35

void setup()

{

Serial.begin(9600);//Set baud rate as 9600

}

void loop()

{

int val;//

int dat;//define variable

val=analogRead(0);// read the analog value and assign to val

dat=(125*val)>>8;//temperature calculation formula

Serial.print("Tep:");//output character string Tep,stand for temperature

Serial.print(dat);//output dat value

Serial.println("C");//output character string C

delay(500);//delay 0.5 s

}

Download the program and open the besel then you can see the temperature.