Arduino Lesson 1

Hello World！

- Intro

Let Arduino say “Hello World!” only with Arduino and USB download cable as below.This experiment allows Arduino to communicate with computer.

- Parts required：

Arduino main board

USB download cable

Start Arduino (download at: http://arduino.cc/en/Main/Software) after arduino driver installed. Program to make Arduino accept  order say “Hello World！”，and you could make it work by simply using “if（）”.Use the “L” led on Arduino to make a blink then display “Hello World！”

- Program for reference.

int val;//Define variable val

int ledpin=13;//Define digital interface 13

void setup()

{

Serial.begin(9600);// Setup Baud Rate as 9600，same as the software setup。When access specific device (such as bluetooth), make sure the baud rate setup consistently.

pinMode(ledpin,OUTPUT);//Setup digital interface Interface13 as output，Arduino all the I/O interface used on Arduino have to be defined like this.

}

void loop()

{

val=Serial.read();//Read the command or characters from PC to Arduino,and assign to val

if(val=='R')//Determine if the command or characters is “R”

{//If receive “R”

digitalWrite(ledpin,HIGH);//Turn on LED on digital interface 13.

delay(500);

digitalWrite(ledpin,LOW);//Turn off LED on digital interface 13.

delay(500);

Serial.println("Hello World!");//Display“Hello World！”characters.

}

}

}