Arduino Lesson 21

PS2 Joystick

PS2 Joystick generally can be used to control the car, and so on, its structure is mainly two 10 k potentiometer and a key switch.

5 Posts are VCC，X，Button，Y，GND。

- Connect-it-up

- Code for reference

- #include <LiquidCrystal.h>   // invoking arduino build-in LiquidCrystal library

- LiquidCrystal lcd(12, 11, 10, 9, 8, 7);//Set up interface

- int xpotPin = 0;  //Set up analog port #0 as signal input port for “x”

- int ypotPin = 1;  // Set up analog port #1 as signal input port for “Y”

- int bpotPin = 2;  // Set up analog port #2 as signal input port for “button”

- int xval=0;    //set up variable

- int yval=0;

- int bval=0;

- void setup()

- {

- lcd.begin(16, 2);  // initialize LCD

- delay(1000); //delay 1000ms

- }

- void loop ()

- {

- xval = analogRead(xpotPin);   //xval variable

is the reading from #0.

- yval = analogRead(ypotPin);   // Yval variable

is the reading from #1.

- bval = analogRead(bpotPin);   //bval variable

is the reading from #2.

- lcd.clear(); //clear

- lcd.setCursor(0, 0) ; //设置光标位置为第一行第一个位置

- lcd.print("X=");      //make lcd display X=

- lcd.print(xval);

- lcd.setCursor(7, 0) ; //设置光标位置为第一行第八个位置

- lcd.print("Y=");      // make lcd display  Y=

- lcd.print(yval);

- if (bval<500)

- {

- lcd.setCursor(0, 1) ;   //设置光标位置为第二行第一个位置

- lcd.print("Button ON"); // make lcd display Button ON

- }

- else

- {

- lcd.setCursor(0, 1) ;

- lcd.print("Button OFF"); // make lcd display  Button OFF

- }

- delay(100);                     //delay 0.1秒

- }

- #include <LiquidCrystal.h>   //调用arduino自带的LiquidCrystal库

- LiquidCrystal lcd(12, 11, 10, 9, 8, 7);//设置接口

- int xpotPin = 0;  // Set up analog port #0 as signal input port for “x”

- int ypotPin = 1;  // Set up analog port #1 as signal input port for “Y”

- int bpotPin = 2;  // Set up analog port #2 as signal input port for “button”

- int xval=0;    // set up variable

- int yval=0;

- int bval=0;

- void setup()

- {

- lcd.begin(16, 2);  //initialize LCD

- delay(1000); //delay 1000ms

- }

- void loop ()

- {

- xval = analogRead(xpotPin);   //xval变量为从0信号口读取到的数值

- yval = analogRead(ypotPin);   //yval变量为从1信号口读取到的数值

- bval = analogRead(bpotPin);   //bval变量为从2信号口读取到的数值

- lcd.clear(); //clear

- lcd.setCursor(0, 0) ; // Set cursor position to the first line of the first position

- lcd.print("X=");      // make the lcd display words X=

- lcd.print(xval);

- lcd.setCursor(7, 0) ; // Set cursor position for the first line of the eighth position

- lcd.print("Y=");      // make the lcd display words Y=

- lcd.print(yval);

- if (bval<500)

- {

- lcd.setCursor(0, 1) ;   // Set cursor position to the second line of the first position

- lcd.print("Button ON"); // make the lcd display words “Button On”

- }

- else

- {

- lcd.setCursor(0, 1) ;

- lcd.print("Button OFF"); //make the lcd display words “Button OFF”

- }

- delay(100);                     //delay 0.1sec，means refresh the speed

- }

Push the joystick, the potentiometer resistance changes, the corresponding voltage change, also the Numbers in the LCD screen is digital output of each shaft voltage. Press the button, button state on the LCD display changes.