Arduino Lesson17

74HC595

- Intro

74HC595 has a 8-bit shifting register , a memorizer and 3 states output. Here we use it control 8 leds. Why? Because you have to use 8 interfaces if you use Arduino. But how many I/O we have on Arduino? Include the analog interface is 20 totally. These 8 leds would take lots of space. So we use 74HC595 to reduce the I/O interface occupation. Now we can use just 3 I/O to control 8 leds.

- Parts Required

74HC595 x 1

Red 5mm LED x 4

Green 5mm LED x 4

220ΩResistors x 8

Breadboard x 1

Jumper wires x 1 Bundle

Schematic diagram

- Program for reference

int data = 2;

int clock = 4;

int latch = 5;

int ledState = 0;

const int ON = HIGH;

const int OFF = LOW;

void setup()

{

pinMode(data, OUTPUT);

pinMode(clock, OUTPUT);

pinMode(latch, OUTPUT);

}

void loop()

{

int delayTime = 100;

for(int i = 0; i < 256; i++)

{

updateLEDs(i);

delay(delayTime);

}

}

void updateLEDs(int value)

{

digitalWrite(latch, LOW);

shiftOut(data, clock, MSBFIRST, value);

digitalWrite(latch, HIGH);

}

void updateLEDsLong(int value)

{

digitalWrite(latch, LOW);

for(int i = 0; i < 8; i++)

{

int bit = value & B10000000;

value = value << 1;

if(bit == 128){digitalWrite(data, HIGH);}

else{digitalWrite(data, LOW);}

digitalWrite(clock, HIGH);

delay(1);

digitalWrite(clock, LOW);

}

digitalWrite(latch, HIGH);

}

int bits[]={B00000001, B00000010, B00000100, B00001000, B00010000, B00100000,

B01000000, B10000000};

int masks[] ={B11111110, B11111101, B11111011, B11110111, B11101111, B11011111,

B10111111, B01111111};

void changeLED(int led, int state)

{

ledState = ledState & masks[led];

if(state == ON){ledState = ledState | bits[led];}

updateLEDs(ledState);

}

Download the program and you can see 8 beautiful blinking leds.