Arduino Lesson 7

Responder

You may finish this experiment easily after you learn the previous lessons. This one actually is an extension for the push-button experiment,use 3 push buttons, 3 leds and 6 digital

I/O interface.

- Connect-it-up

- Program for reference

int redled=10;

int yellowled=9;

int greenled=8;

int redpin=7;

int yellowpin=6;

int greenpin=5;

int red;

int yellow;

int green;

void setup()

{

pinMode(redled,OUTPUT);

pinMode(yellowled,OUTPUT);

pinMode(greenled,OUTPUT);

pinMode(redpin,INPUT);

pinMode(yellowpin,INPUT);

pinMode(greenpin,INPUT);

}

void loop()

{

red=digitalRead(redpin);

if(red==LOW)

{ digitalWrite(redled,LOW);}

else

{ digitalWrite(redled,HIGH);}

yellow=digitalRead(yellowpin);

if(yellow==LOW)

{ digitalWrite(yellowled,LOW);}

else

{ digitalWrite(yellowled,HIGH);}

green=digitalRead(greenpin);

if(green==LOW)

{ digitalWrite(greenled,LOW);}

else

{ digitalWrite(greenled,HIGH);}

}

There is no difference between the push button experiment and

One except more parts been used. Download and your responder

is ready!