Arduino Lesson 2

Let them blink!

- Intro

LED experiment is a basic one，last lesson“ Hello World！”already used Arduino build-in LED，this time we use other I/O interface and external DIP LED.

- Connection

Besides Arduino and USB cable, we also need below parts.

Red 5mm DIP LED x 1

220ΩResistor x 1

Breadboard x 1

Jumper wires x 1 Bundle

Now We follow schematic diagram and physical figure below to connect parts.We use digital interface #10.A current-limit resistor MUST be used here in case the LED burn out.

Now we start to program after all parts connected. First we make led

turn on for 1 sec.and turn off 1 sec. This is as simple as last one, just

switch the interface #13 to #10.

- Program for reference.

int ledPin = 10; //Define digital interface #10.

void setup()

{

pinMode(ledPin, OUTPUT);// Define led interface as output.

}

void loop()

{

digitalWrite(ledPin, HIGH); //Turn on the led.

delay(1000); //Delay 1 sec.

digitalWrite(ledPin, LOW); //Turn off the led.

delay(1000); // Delay 1 sec.

}

Download the program you will see the led flickering, so our small lights flashing experiment  is done.