Arduino Lesson 6

Push Button

I/O means INPUT interface and OUTPUT interface，so far the

LED experiment we designed just use the I/O output function

of Arduino. This experiment we will try to use the input function(i.e read the value from external device).

We use a push button and a led light to make an experiment use both input and output to let you know more about I/O.

- Parts Required

Push button x 1

Red 5mm LED x 1

220ΩResistor x 1

10KΩResistor x 1

Breadboard x 1

Jumper wires x 1 Bundle

- Connect-it-up

Access the button to digital interface #7, red led light to

#11.(I/O interface #0-#13 on Arduino all can be used to connect

the button and led,but try NOT to use #0 and #1 as they are multiuse interfaces, also the serial communication port. So keep them available to avoid any extra un-plug operation during experiment).

Now we begin to program to lighten the led when button pushed.

This one just add one more condition determine sentence (if

sentence)than the previous lessons. Arduino program base on C

language, so sentence from C can be used in Arduino, like while,

switch etc.

The below demo is a simple if sentence.

We can know that the #7 interface value read high level when

button is pushed according to the wiring.At the meantime, we

will turn on the light when #11 interface output high level.

In the program, we determine #7 interface is low level, if

it was then also need to make #11 as low level too.

- Program for reference

int ledpin=11;//define digital interface #11

int inpin=7;// define digital interface #7

int val;//define variable val

void setup()

{

pinMode(ledpin,OUTPUT);//define led interface as output

pinMode(inpin,INPUT);//define button interface as input

}

void loop()

{

val=digitalRead(inpin);//read digital interface #7 and assign

the level value to val

if(val==LOW)//test if the button been pushed,if did then

light on

{ digitalWrite(ledpin,LOW);}

else

{ digitalWrite(ledpin,HIGH);}

}

Download the program and your experiment is finish.You can

find this type of application very widely use in daily life.For

example, when you push your cellphone’s main button and

backlight is on. The next one we are going to make another common

one—Responder.