Arduino Lesson 5

Traffic Light

- Intro

- This experiment actually is an extend version of lesson 2.

- We can use lesson 2 for reference, use digital interface # 10、#7、#4.

- Use Arduino delay（）function to control the delay time.

- Parts required

Besides Arduino main board and USB cable, we also need

Red 5mm LED x 1

Yellow 5mm LED x 1

Green 5mm LED x 1

220ΩResistor x 3

Breadboad x 1

Jumper wires x 1 Bundle

- Program for reference

int redled =10; // Define digital interface #10.

int yellowled =7; // Define digital interface #7.

int greenled =4; // Define digital interface #4.

void setup()

{

pinMode(redled, OUTPUT);// Define red LED interface as output.

pinMode(yellowled, OUTPUT); // Define yellow LED interface as output.pinMode(greenled, OUTPUT); // Define green LED interface as output.

}

void loop()

{

digitalWrite(redled, HIGH);// Turn on red led.

delay(1000);//Delay 1 sec

digitalWrite(redled, LOW); // Turn off red led.

digitalWrite(yellowled, HIGH);// Turn on yellow led.

delay(200);//Delay 0.2 sec

digitalWrite(yellowled, LOW);//Turn off yellow led.

digitalWrite(greenled, HIGH);// Turn on the green led.

delay(1000);//Dealy 1 sec.

digitalWrite(greenled, LOW);//Turn off green led.

}

Download the program you will see the traffic light is working.