Arduino Lesson 16

LED 4 Digit. 7-Segment Display

- Intro

This experiments we use the arduino to drive a four digital common anode display. Current limit resistor is a must have, there are two types of current limiting resistor connection, one is in d1 - d4 anode, a total of four. This type requires less resistors, but will show different brightness numbers on each position. #1 is brightest, #8 is the most dark. The other type is connecting with the other 8 pins. This type gives average brightness, but need more resistors. Here we use 8pcs 220Ω resistors.

4-digit display has 12 pins. When the dot down and stay put, left bottom one is #1, the others are in counterclockwise order,top left corner one is #12.

Below picture is the instruction.

- Connect-it-up

- ARDUINO CODE

- //set the cathode interface

- int a = 1;

- int b = 2;

- int c = 3;

- int d = 4;

- int e = 5;

- int f = 6;

- int g = 7;

- int p = 8;

- // set the anode interface

- int d4 = 9;

- int d3 = 10;

- int d2 = 11;

- int d1 = 12;

- //set variable

- long n = 0;

- int x = 100;

- int del = 55;  //fine-tuning value for clock

- void setup()

- {

- pinMode(d1, OUTPUT);

- pinMode(d2, OUTPUT);

- pinMode(d3, OUTPUT);

- pinMode(d4, OUTPUT);

- pinMode(a, OUTPUT);

- pinMode(b, OUTPUT);

- pinMode(c, OUTPUT);

- pinMode(d, OUTPUT);

- pinMode(e, OUTPUT);

- pinMode(f, OUTPUT);

- pinMode(g, OUTPUT);

- pinMode(p, OUTPUT);

- }

- void loop()

- {

- clearLEDs();

- pickDigit(1);

- pickNumber((n/x/1000)%10);

- delayMicroseconds(del);

- clearLEDs();

- pickDigit(2);

- pickNumber((n/x/100)%10);

- delayMicroseconds(del);

- clearLEDs();

- pickDigit(3);

- dispDec(3);

- pickNumber((n/x/10)%10);

- delayMicroseconds(del);

- clearLEDs();

- pickDigit(4);

- pickNumber(n/x%10);

- delayMicroseconds(del);

- n++;

- if (digitalRead(13) == HIGH)

- {

- n = 0;

- }

- }

- void pickDigit(int x)  //define pickDigit(x),to open dx port

- {

- digitalWrite(d1, LOW);

- digitalWrite(d2, LOW);

- digitalWrite(d3, LOW);

- digitalWrite(d4, LOW);

- switch(x)

- {

- case 1:

- digitalWrite(d1, HIGH);

- break;

- case 2:

- digitalWrite(d2, HIGH);

- break;

- case 3:

- digitalWrite(d3, HIGH);

- break;

- default:

- digitalWrite(d4, HIGH);

- break;

- }

- }

- void pickNumber(int x)   //define pickNumber(x)to display number x

- {

- switch(x)

- {

- default:

- zero();

- break;

- case 1:

- one();

- break;

- case 2:

- two();

- break;

- case 3:

- three();

- break;

- case 4:

- four();

- break;

- case 5:

- five();

- break;

- case 6:

- six();

- break;

- case 7:

- seven();

- break;

- case 8:

- eight();

- break;

- case 9:

- nine();

- break;

- }

- }

- void dispDec(int x)  //set to start the decimal point

- {

- digitalWrite(p, LOW);

- }

- void clearLEDs()  //clear contents

- {

- digitalWrite(a, HIGH);

- digitalWrite(b, HIGH);

- digitalWrite(c, HIGH);

- digitalWrite(d, HIGH);

- digitalWrite(e, HIGH);

- digitalWrite(f, HIGH);

- digitalWrite(g, HIGH);

- digitalWrite(p, HIGH);

- }

- void zero()  //define 0 as cathode pin switch

- {

- digitalWrite(a, LOW);

- digitalWrite(b, LOW);

- digitalWrite(c, LOW);

- digitalWrite(d, LOW);

- digitalWrite(e, LOW);

- digitalWrite(f, LOW);

- digitalWrite(g, HIGH);

- }

- void one()  // define 1 as cathode pin switch

- {

- digitalWrite(a, HIGH);

- digitalWrite(b, LOW);

- digitalWrite(c, LOW);

- digitalWrite(d, HIGH);

- digitalWrite(e, HIGH);

- digitalWrite(f, HIGH);

- digitalWrite(g, HIGH);

- }

- void two()  // define 2 as cathode pin switch

- {

- digitalWrite(a, LOW);

- digitalWrite(b, LOW);

- digitalWrite(c, HIGH);

- digitalWrite(d, LOW);

- digitalWrite(e, LOW);

- digitalWrite(f, HIGH);

- digitalWrite(g, LOW);

- }

- void three()  // define 3 as cathode pin switch

- {

- digitalWrite(a, LOW);

- digitalWrite(b, LOW);

- digitalWrite(c, LOW);

- digitalWrite(d, LOW);

- digitalWrite(e, HIGH);

- digitalWrite(f, HIGH);

- digitalWrite(g, LOW);

- }

- void four()  // define 4 as cathode pin switch

- {

- digitalWrite(a, HIGH);

- digitalWrite(b, LOW);

- digitalWrite(c, LOW);

- digitalWrite(d, HIGH);

- digitalWrite(e, HIGH);

- digitalWrite(f, LOW);

- digitalWrite(g, LOW);

- }

- void five()  // define 5 as cathode pin switch

- {

- digitalWrite(a, LOW);

- digitalWrite(b, HIGH);

- digitalWrite(c, LOW);

- digitalWrite(d, LOW);

- digitalWrite(e, HIGH);

- digitalWrite(f, LOW);

- digitalWrite(g, LOW);

- }

- void six()  // define 6 as cathode pin switch

- {

- digitalWrite(a, LOW);

- digitalWrite(b, HIGH);

- digitalWrite(c, LOW);

- digitalWrite(d, LOW);

- digitalWrite(e, LOW);

- digitalWrite(f, LOW);

- digitalWrite(g, LOW);

- }

- void seven()  // define 7 as cathode pin switch

- {

- digitalWrite(a, LOW);

- digitalWrite(b, LOW);

- digitalWrite(c, LOW);

- digitalWrite(d, HIGH);

- digitalWrite(e, HIGH);

- digitalWrite(f, HIGH);

- digitalWrite(g, HIGH);

- }

- void eight()  // define 8 as cathode pin switch

- {

- digitalWrite(a, LOW);

- digitalWrite(b, LOW);

- digitalWrite(c, LOW);

- digitalWrite(d, LOW);

- digitalWrite(e, LOW);

- digitalWrite(f, LOW);

- digitalWrite(g, LOW);

- }

- void nine()  // define 9 as cathode pin switch

- {

- digitalWrite(a, LOW);

- digitalWrite(b, LOW);

- digitalWrite(c, LOW);

- digitalWrite(d, LOW);

- digitalWrite(e, HIGH);

- digitalWrite(f, LOW);

- digitalWrite(g, LOW);

- }

Copy the code and download to Arduino and see the effects.