Arduino Lesson 13

Tilt Switch Control Light

- Parts requred

- Mercury Tilt Switch x 1

- Led Lightx 1

- 220ΩResistor x 1

- Jumper wires x 1 Bundle

- Connect-it-up

Connect the Arduino,extension board,breadboard and USB download cable as Arduino tutorial written,access led to digital pin #8,tilt switch to analog pin #5.

- Theory

Switch and led light on when one side of switch lower than horizontal position，analog voltage around 5v( Binary is 1023),switch and light off when the other side of mercury switch lower tan horizontal position,analog voltage around 0v(Binary is 0).Use a if function to set if the analog voltage >2.5v(Binary is 512)to know the switch on or off.

- Program for reference void setup() { pinMode(8,OUTPUT);//set digital pin #8 in output mode } void loop() { int i;//Define variable i while(1) { i=analogRead(5);//读Read the voltage of analog interface #5 if(i>200)//if bigger than 512（2.5V） { digitalWrite(8,HIGH);//light up led } else//otherwise { digitalWrite(8,LOW);//turn off led } } }

Download program to Arduino and tilt the board to see the status of the light.